# SafeNav-LLM: Semantic Waypoint Resolution via On-Device Language Model as a Native Nav2 BehaviorTree Plugin

## 1. Knowledge on Domain / Problem Statement

### 1.1 The Deployment Reality of ROS 2 Navigation

The Robot Operating System 2 has matured into the dominant middleware for indoor mobile robotics. Its navigation stack, Nav2, provides a production-grade pipeline combining occupancy-grid mapping, AMCL localisation, global path planning (Smac Planner, NavFn), local trajectory control (DWB, TEB, RPP), costmap management, and a BehaviorTree-based mission executive. This architecture is powerful and extensible, but it imposes a fundamentally technical interface on every operator: to command a robot to go somewhere, you must supply a geometry_msgs/PoseStamped: a metric (x, y, θ) pose expressed in the robot's coordinate frame, submitted through RViz2's graphical tool or a ROS action client.

For a robotics engineer, this is natural. For a nurse asking a hospital delivery robot to bring medication from storage, a warehouse supervisor redirecting a TurtleBot to Bay 7, or a lab technician telling an assistant robot to fetch equipment from the charging station, it is an unreasonable technical burden. These operators think in terms of named places, not coordinate tuples. The gap between how humans naturally describe navigation goals and what Nav2 actually accepts is the core problem this project addresses.

### 1.2 Why Large Language Models Are Not Enough on Their Own

Large language models can parse arbitrarily phrased navigation commands with high reliability. "Go to the charging station near the fire exit," "head to where we keep the reagents," "not the main lab, the one next to it": current models handle spatial language, negation, indirect reference, and ambiguity well. The naive solution is obvious: feed the command to an LLM, get a goal, send it to Nav2.

The problem is that LLMs hallucinate. A model that does not know the specific layout of your building will confidently return a plausible-sounding but geometrically wrong output: a room name that doesn't exist on this floor, coordinates outside the navigable map, or a pose on the wrong side of a wall. For a robot weighing 20 kg moving at 0.5 m/s, a hallucinated goal is not an inconvenience. It is a collision event or an unrecoverable navigation failure that requires physical human intervention to resolve.

Every existing LLM-navigation system handles this in one of two ways: it accepts the hallucination risk (open-vocabulary systems like VLMaps and NLMap, which deliberately remove constraints), or it relies on post-hoc validation (SPINE uses a separate validator to check LLM outputs after generation). Neither approach is architecturally safe; both allow the LLM to generate an invalid goal, then try to catch it afterward.

### 1.3 The Three-Dimensional Problem

This project addresses three simultaneous requirements that no existing system meets together:

Safety. The language model must be structurally incapable of producing any navigation goal that is not in the robot's pre-validated location database. Not discouraged by a prompt instruction. Not filtered by a post-hoc validator. Structurally impossible: prevented at the token-sampler level so that the constraint holds regardless of what the user says or how the model's weights respond to unusual inputs.

Deployment independence. The entire resolution pipeline must run on the robot's onboard compute with no internet connection. Hospital wards, factory floors, cleanrooms, and secure research facilities frequently have no outbound internet access. A system that calls the OpenAI API for every navigation command is not deployable in these environments. Beyond connectivity, cloud dependency introduces per-call monetary cost, response latency variability tied to network conditions, and a data-privacy issue: command strings describing robot locations inside a facility leave the building.

Native ROS 2 integration. The LLM resolution must be a first-class participant in the Nav2 BehaviorTree execution graph, not an external preprocessing step. Existing systems interface with Nav2 from outside: they call the Nav2 action server as a client, but do not participate in Nav2's tick timing, lifecycle management, blackboard communication, or fallback/recovery behavior trees. A native BT plugin can participate in all of these, enabling graceful failure handling, retry logic, and safe fallback behaviors through standard BT mechanisms rather than custom wrapper code.

### 1.4 The Core Innovation: Annotation Graph with Grammar-Constrained Decoding

SafeNav-LLM resolves all three dimensions through a single architectural decision. Rather than asking the LLM to generate a navigation goal, the system asks it to select a goal from a closed, human-curated list of named locations, the annotation graph. The LLM's output vocabulary is constrained at the token-sampler level using llama.cpp's GBNF (Grammar-Based Normal Form) grammar API, so the model is physically unable to produce any room name or coordinate not present in the validated graph. Room names in the graph map deterministically to metric poses, so the coordinate lookup is a dictionary operation, not an LLM generation step.

This closed-set constraint is implemented inside a BehaviorTree.CPP action node, registered as a Nav2 plugin via pluginlib, making LLM resolution an in-tree operation. A confidence calibration model, trained on the robot's own deployment logs, gates goal emission and redirects low-confidence resolutions to a human clarification request.

### 1.5 Assumptions and Constraints

The environment is semi-structured and mapped. Room locations change infrequently enough that a one-time human annotation session (approximately 20 minutes for 10 rooms) is acceptable. The Jetson Orin NX 8GB provides sufficient compute for Phi-3 mini Q4_K_M inference at interactive latency. Commands are in English.

Hard constraints: on-device inference must complete within the configurable BT tick timeout (default 5 seconds); the annotation graph must be maintainable without robotics expertise; the confidence gate must achieve a false-rejection rate below 15% and a false-pass rate below 5% on the held-out evaluation set; the package must install via colcon build on any ROS 2 Humble system without modifying Nav2 source code.

## 2. Literature Review

### Cluster A: Foundational LLM-Navigation Systems

[1] LM-Nav, Shah, Osiński, Ichter, Levine (CoRL 2022, arXiv:2207.04429). The first end-to-end system chaining GPT-3, CLIP-ViT, and the learned visual navigation model ViNG for outdoor robot navigation. GPT-3 parses a natural-language command and extracts an ordered sequence of visual landmark descriptions. CLIP-ViT matches those descriptions to nodes in a topological graph constructed from prior traversal video. The robot navigates between matched landmark nodes using ViNG. The landmark graph implicitly constrains LLM output: it can only navigate to landmarks already in the graph, making LM-Nav conceptually adjacent to the annotation graph idea. Critical gaps: GPT-3 runs via OpenAI API (cloud-dependent, no offline mode); the topological graph is built automatically from prior video traversal, not human-annotatable for new environments; no confidence calibration is produced; designed for GPS-scale outdoor navigation, not indoor metric precision.

[2] SayCan / PaLM-SayCan, Ahn et al., Google (arXiv:2204.01691, Science Robotics 2022). Established the dominant LLM-robot planning paradigm: an LLM scores which of an enumerated closed skill set is task-relevant (task-grounding score), while a learned affordance value function scores which skills are physically feasible given current robot state (world-grounding score). The product of the two scores selects the action. SayCan's skill repertoire includes 551 primitives across seven families, including "navigate to [hard-coded location]," making the navigation skill set effectively a closed named-location list, conceptually related to the annotation graph. Achieves 84% plan selection accuracy and 74% execution success on kitchen tasks. Gaps: runs PaLM on Google infrastructure (cloud-only, no embedded variant); the skill set is defined by trained robot policies, not a human annotation file; targets kitchen manipulation, not navigation as the primary task. The SayCan patent family, Google's US20230311335A1 / WO2023192497A1, is the primary freedom-to-operate concern for this project.

[3] NLMap, Chen, Xia, Ichter et al., Google (arXiv:2209.09874, ICRA 2023). Explicitly built to overcome SayCan's fixed skill repertoire. NLMap constructs an open-vocabulary queryable 3D scene representation so the robot can plan "without a fixed list of objects nor executable options." Instructive as counter-prior-art: the authors treat the closed set as a defect. SafeNav-LLM deliberately re-imposes the closed set as a safety mechanism, a conceptual inversion that strengthens the inventive step argument because a prominent prior-art paper argues the opposite direction.

[4] SayPlan, Rana et al., QUT (arXiv:2307.06135, CoRL 2023). Uses an LLM to perform semantic search over a hierarchical 3D scene graph for long-horizon task planning. The LLM queries graph nodes for relevant objects and locations, and iterative replanning is validated against a scene-graph simulator. Operates at semantic object-level planning rather than metric navigation goal resolution. No confidence calibration; no BehaviorTree integration; cloud LLM.

[5] VLMaps, Huang, Mees, Zeng, Burgard (arXiv:2210.05714, ICRA 2023). Fuses visual-language model features into a 3D map, enabling LLM-translated natural-language spatial goals to be localised in the map (e.g., "to the left of the sofa"). Fundamentally open-vocabulary, with no closed-set constraint, which creates hallucination risk in environments with unseen objects. No ROS 2 or Nav2 integration; cloud VLM and LLM.

### Cluster B: ROS 2 / Nav2 Integration

[6] Latency-Aware LLM Benchmarking for ROS 2 Nav, Das, Hussain & Nawaz (Sensors 2026, DOI: 10.3390/s26020608). The most directly comparable published work. Benchmarks eight LLMs (GPT-3.5, GPT-4, GPT-5, Claude 3.7, Gemini 2.5, Mistral-7B, DeepSeek-R1, LLaMA-3.3-70B) integrated with ROS 2 Nav2 on a simulated TurtleBot4 in Gazebo Fortress. Measures end-to-end latency, instruction-parsing accuracy, path quality, and task success rate across DWB, TEB, and RPP local planners. Full code available on GitHub. This is the headline prior art for the same problem. Gaps: all eight models run via external APIs (no on-device inference); LLM generates free-form navigation intent with no closed-set constraint; no confidence calibration; integration is a Nav2 action-server wrapper, not a native BT plugin.

[7] LLM + ROS2 + SLAM for Natural Language Navigation, Pascual Espada et al. (SAGE Journals, 2025). Integrates LLMs with ROS2, SLAM, and Nav2 for natural-language navigation including autonomous exploration. Explicitly names the gap this project addresses: "current approaches are not equipped to effectively translate contextual references (e.g. next to the corner, between doors) into concrete robot commands." Cloud LLM; no closed-set constraint; no confidence calibration.

[8] osmAG Navigation Copilot, Xie, Schwertfeger (arXiv:2409.08493, 2024). Uses an LLM as a navigation "copilot" over an osmAG textual map with ROS move_base. A PassageCostEvaluator selects a named destination from the osmAG node list. Architecturally closer than most: the LLM is constrained to nodes in the map, but it integrates with ROS 1 move_base, not ROS 2 Nav2 BehaviorTree. No confidence calibration; no pluginlib integration; no safety-boundary framing.

### Cluster C: BehaviorTree and LLM Coupling

[9] BTGenBot, Izzo, Bardaro & Matteucci, Politecnico di Milano (arXiv:2403.12761, IROS 2024). LoRA-fine-tunes Llama2-7b, Llama-Chat, and CodeLlama-7b on approximately 600 behavior trees to generate BehaviorTree XML from natural-language commands, running on a Jetson AGX Orin. A "MoveTo" leaf node maps a location name to coordinates via a YAML file and sends the goal to Nav2. The YAML-to-coordinate lookup is structurally analogous to the annotation graph. Key difference: the LLM is an external XML generator, running before the BT tree is instantiated, not an inference engine inside a running BT action node. It cannot participate in BT tick timing, blackboard communication, Nav2 lifecycle management, or fallback tree propagation. No confidence calibration; no token-level closed-set constraint.

[10] Structured BTs + LLMs via ROSA, (arXiv:2508.09621, 2025). GPT-4o interprets commands through ROSA (a ROS-LLM bridge) and selects BT behavior implementations. Active BT nodes are logged for interpretability. Cloud LLM; the LLM is external to the BT execution graph; no pluginlib integration; no safety constraint.

### Cluster D: Confidence, Calibration, and Safety

[11] KnowNo, Ren et al., Princeton / Google DeepMind (arXiv:2307.01928, CoRL 2023). Applies conformal prediction over a calibration dataset to give LLM robot planners a statistical task-completion guarantee. If the prediction set at a given confidence level is not a singleton, the robot asks a human for help. Explicitly targets LLMs' "confidently hallucinated predictions." This is the most direct prior art for the confidence gate. Difference: KnowNo uses out-of-the-box conformal prediction "without model-finetuning," calibrated on a general task distribution. SafeNav-LLM trains a supervised Platt-scaled calibration model on the robot's own deployment logs, capturing the specific phrasing patterns, room name frequencies, and disambiguation challenges of one deployment site. This per-deployment calibration is the distinguishing contribution.

[12] SPINE, Ravichandran, Murali, Tzes, Pappas, Kumar (arXiv:2410.03035, ICRA 2025). A receding-horizon planner that uses an LLM to reason about navigation subtasks. Tasks are "automatically validated for safety and refined online with new map observations." Notes that LLMs "may hallucinate these details." Implements safety through a post-hoc validator: it lets the LLM generate freely, then checks. SafeNav-LLM's grammar constraint is a structural pre-generation restriction, not a post-generation check, making it strictly stronger.

[13] SafeDec, (arXiv:2509.01728, 2025). Applies constrained decoding to transformer navigation policies, enforcing Signal Temporal Logic (STL) safety specifications at the token-sampler level at runtime without retraining. Related to SafeNav-LLM's GBNF grammar constraint in mechanism (both operate at the sampler), but targets low-level motion policies, not semantic goal resolution, and uses formal STL specifications rather than a named-node closed set.

[14] CANVAS, Choi et al. (arXiv:2410.01273, ICRA 2025). Converts continuous waypoints into 128 discrete tokens via K-means clustering, reframing navigation as classification over a learned codebook. Related in spirit (discretising the output space), but the constraint is geometric (cluster centroids), not semantic (named human-readable locations), and the codebook is learned, not human-curated. No LLM integration; no ROS 2 / Nav2 deployment.

### Cluster E: On-Device LLM Inference for Robotics

[15] Plug-and-Play LLM Knowledge Extraction, (MDPI Machine Learning & Knowledge Extraction, 2026, DOI: 10.3390/make8020049). Runs DistilGPT-2 fully on-device on a Jetson AGX Orin with no cloud call, augmenting a finite-state machine with asynchronous velocity suggestions gated by deadline, schema, and kinematic validation. Deterministic 50 Hz control loop with under 5ms fallback. Directly establishes the feasibility of on-device LLM inference for real-time robotics, and provides a latency reference point. Does not address semantic named-location resolution or Nav2 BT integration.

[16] LocalNav, (arXiv:2606.27871, 2026). Distils a frontier vision-language model into a 4B on-device VLM for object-goal navigation on Jetson Orin NX, with no cloud connectivity. Demonstrates that large-model knowledge can be compressed into edge-deployable models without catastrophic accuracy loss on this class of task. Targets open-vocabulary object-goal navigation (find the chair), not semantic named-location resolution (go to the charging dock); no closed-set constraint; no Nav2 BT integration.

[17] osmAG-LLM, Xie, Schwertfeger et al. (arXiv:2507.12753, submitted July 2025). The most structurally similar prior system. Zero-shot open-vocabulary object navigation via an LLM restricted to named nodes of a simplified osmAG graph: the LLM receives only node names and hierarchical relationships, explicitly excluding coordinates and polygons. JSON output is constrained to named nodes. This is close to the annotation graph concept. Distinguishing features: their motivation is representation scalability, not hallucination safety; no safety-boundary framing in the paper; no Nav2 BehaviorTree pluginlib integration (uses move_base); no confidence calibration gate; constraint is enforced via prompt instruction, not at the token-sampler level.

Prior Art Gap Matrix

| Reference | On-device LLM | Closed-set safety constraint | Native Nav2 BT plugin | Deployment-log calibration |
| --- | --- | --- | --- | --- |
| LM-Nav [1] | ✗ (cloud GPT-3) | Partial (landmark graph, not safety-framed) | ✗ | ✗ |
| SayCan [2] | ✗ (cloud PaLM) | Partial (skill repertoire, not location graph) | ✗ | ✗ |
| NLMap [3] | ✗ | ✗ (explicitly open-vocab) | ✗ | ✗ |
| Das et al. [6] | ✗ (8 cloud APIs) | ✗ | ✗ | ✗ |
| BTGenBot [9] | Partial (Jetson, fine-tuned) | ✗ | ✗ (external generator) | ✗ |
| KnowNo [11] | ✗ | ✗ | ✗ | Partial (conformal, not deployment-log) |
| SPINE [12] | ✗ | Partial (post-hoc validator) | ✗ | ✗ |
| osmAG-LLM [17] | ✗ | Partial (prompt-level, not grammar) | ✗ | ✗ |
| LocalNav [16] | ✓ | ✗ | ✗ | ✗ |
| SafeNav-LLM | ✓ | ✓ (grammar-level, structural) | ✓ | ✓ |

No single reference, and no obvious two-reference combination found in the search across 25 papers and 8 patents, anticipates all four columns simultaneously.

## 3. Design of Proposed Methodology

### 3.1 Methodological Principle

The methodology is built on one hard rule: validate the algorithm entirely in simulation before writing a line of hardware-deployment code. This prevents the most costly failure mode in embedded robotics research: discovering a fundamental algorithmic problem only after hardware trials have consumed the project timeline. Each phase produces a concrete gating deliverable; Phase N+1 does not begin until Phase N's deliverable passes its acceptance criteria.

### 3.2 Phase 1: Simulation, Prompt Engineering, and Benchmark Generation

Environment. ROS 2 Humble + Gazebo Harmonic + TurtleBot4 URDF in a custom 10-room Gazebo world modelled on the project lab. SLAM Toolbox builds the occupancy grid; AMCL provides localisation during navigation. The annotation graph is created using the annotate_map.py CLI tool, which listens on /initialpose and /clicked_point, the same RViz2 topics used by the 2D Nav Goal tool. The operator clicks each room's docking pose in RViz2; the CLI prompts for canonical name, aliases, and semantic tags; the tool writes room_annotations.json. Total annotation time for 10 rooms: approximately 20 minutes.

LLM resolver: development and prompt sensitivity study. The llm_resolver_node is prototyped in Python (rclpy) for rapid iteration, then ported to C++ (rclcpp) for the final implementation. Phi-3 mini Q4_K_M is served via llama.cpp's C API, with the model context held warm across service calls to eliminate per-call loading overhead (~2.4 GB VRAM, leaving ~5.6 GB available on the Orin NX 8GB).

A full prompt sensitivity study is run before any hardware work: few-shot count ∈ {0, 5, 10, 20} × temperature ∈ {0.0, 0.1, 0.3} × output format ∈ {free JSON, GBNF grammar-constrained JSON}. Each of the 18 combinations is evaluated on the 500-pair synthetic benchmark. Top-1 accuracy and p90 latency are the selection criteria. The grammar-constrained condition also reports out-of-graph output rate (expected: 0% structurally; any non-zero rate indicates a grammar implementation bug).

Synthetic benchmark dataset. 500 command–room pairs generated via the GPT-4 API, covering eight phrasing categories with manual review:

| Category | Example | Expected output |
| --- | --- | --- |
| Direct naming | "go to the kitchen" | kitchen |
| Alias use | "head to the pantry" | kitchen |
| Spatial reference | "the room near the main entrance" | reception |
| Functional description | "somewhere I can charge the robot" | charging_dock |
| Negation | "not the storage room, the other one" | server_room |
| Abbreviation | "server rack area" | server_room |
| Multi-hop | "the room left of the meeting room" | lab_bench_1 |
| Adversarial | "go to the cafeteria" (does not exist) | REJECT |

Gating deliverable: prompt configuration selected (expected: temperature=0.1, 5 few-shot, grammar-constrained); benchmark accuracy baseline ≥ 80% top-1 in simulation.

### 3.3 Phase 2: BehaviorTree Plugin and Nav2 Integration

bt_semantic_plugin. A BT::ActionNodeBase subclass with one input port (command: string) and two output ports (resolved_pose: geometry_msgs/PoseStamped, resolved_room: string). The tick() method:

- Retrieves command from the blackboard input port.

- Calls /list_rooms service to get the current candidate name list (atomic call, ~0.1ms).

- Calls /resolve_waypoint service via rclcpp::Client async future, blocking with configured timeout_ms.

- If service times out: returns NodeStatus::FAILURE immediately (deterministic, no hang).

- Applies the calibration model (serialised sklearn pipeline, 5-feature logistic regression): if calibrated_confidence ≥ threshold, writes pose to blackboard, returns NodeStatus::SUCCESS.

- If below threshold: publishes to /clarification_request, returns NodeStatus::FAILURE. The BT fallback subtree executes a SpeakText action (espeak-ng) listing known room names and prompting the operator to rephrase.

The plugin is registered in plugin_description.xml and added to bt_navigator.plugin_lib_names in nav2_params.yaml. A custom BT XML file routes SemanticNavigateToPose into the standard NavigateToPose action.

GBNF grammar construction. The grammar is generated dynamically from the annotation graph's canonical names:

root ::= "{" ws "\"room\"" ws ":" ws room-value "," ws "\"confidence\"" ws ":" ws confidence-value ws "}"

room-value ::= "\"charging_dock\"" | "\"kitchen\"" | "\"server_room\"" | ...

confidence-value ::= [0-9] "." [0-9] [0-9]

This grammar is passed to llama_grammar_init at each service call. It is regenerated whenever AddRoom.srv is called. The constraint is at the token sampler, not a regex applied post-generation, making it structurally impossible to produce an out-of-graph output regardless of prompt content or model behavior.

End-to-end simulation validation. 150 runs (10 rooms × 5 phrasing variants × 3 trials). Logged via ros2 bag. Each run records: command string, resolved room, calibrated confidence, PoseStamped goal, navigation outcome, end-to-end latency. Ablation: grammar-on vs. grammar-off on same 150 commands. Expected result: 0 out-of-graph outputs with grammar-on; measurable non-zero out-of-graph rate with grammar-off.

Gating deliverable: all 150 sim runs with grammar-on produce zero out-of-graph outputs; p90 latency < 3,000ms in simulation; ablation shows grammar-off produces ≥ 5% out-of-graph rate.

### 3.4 Phase 3: Hardware Deployment and Calibration Model Training

Physical setup. TurtleBot4 Standard (iRobot Create3 base, Jetson Orin NX 8GB, OAK-D Pro camera, RPLiDAR A2M8). 10-room lab annotated at 30 named locations, with intentionally overlapping aliases for disambiguation stress testing. Fresh SLAM map built at the start of each session week.

Session 1: calibration training set (200 trials). Five non-expert participants (2 administrative staff, 2 non-robotics PhD students, 1 undergraduate from another department) each issue 40 commands across all 30 locations. Participants are told the robot knows 30 locations but not given the exact naming conventions; they use natural phrasing. Outcomes (correct room reached / wrong room / command rejected) are recorded.

Calibration model training. Five features per trial:

- Raw LLM-reported confidence (from the model's verbalized "confidence" field in the JSON output)

- Output token entropy (computed from llama.cpp's logprob output, measures how "uncertain" the model's token distribution was)

- Number of candidate rooms in the annotation graph (larger candidate sets increase ambiguity)

- Command character length (longer commands tend to be more ambiguous or multi-hop)

- Minimum Levenshtein edit distance between the resolved room name and any substring of the command (measures how lexically close the command was to a known name)

Training: Platt-scaled logistic regression on Session 1 binary outcomes (correct/incorrect), 5-fold leave-one-participant-out cross-validation. Threshold set at the point achieving 5% false-pass rate on Session 1 training set.

Session 2: evaluation set (100 trials). Same five participants, minimum one week later, 20 commands each. Session 2 data is entirely held out, with no overlap with Session 1. All reported metrics drawn exclusively from Session 2.

Gating deliverable: calibration model with Session 1 5-fold CV ECE ≤ 0.10; Session 2 false-pass rate ≤ 5%, false-rejection rate ≤ 15%.

### 3.5 Phase 4: Evaluation, Ablation, and Dissemination

Full metric collection on Session 2. Three-condition experiment (Condition A: metric baseline, Condition B: full system, Condition C: no calibration gate). Ablation across four model sizes (Phi-3 mini Q4_K_M, TinyLlama Q4, Phi-3.5 mini Q8, cloud GPT-4o as ceiling). Paper draft for RA-L or ICRA 2026. Indian provisional patent application (IPO Form 2A, ₹1,750 educational rate).

## 4. Module Description / System Design

### 4.1 Four-Node Architecture

The system is a single ROS 2 ament_cmake package (semantic_waypoint_planner) consisting of four nodes, each with one responsibility, communicating through typed interfaces. No node shares memory with another; all communication is through ROS topics, services, and actions. The entire package installs without modifying Nav2 source; it activates via a one-line addition to nav2_params.yaml.

### 4.2 Module 1: /semantic_goal_node

Type: rclcpp Lifecycle Node.

Responsibility: the human-facing front end. Accepts SemanticGoal.msg from an operator interface: either a web dashboard via rosbridge_suite, a voice-recognition node (Whisper-ROS), or a simple ROS topic publisher for testing. Validates the message (non-empty command, confidence threshold in range, valid frame_id), then forwards to the BT navigator via a Nav2 NavigateToPose action call. Deliberately thin, it contains no inference logic so the resolution pipeline can be tested in isolation.

Interfaces: subscribes to /semantic_goal [SemanticGoal]; calls /navigate_to_pose [NavigateToPose] action server.

Key parameter: default_confidence_threshold: 0.72 (overridden per-call by message field).

### 4.3 Module 2: /llm_resolver_node

Type: rclcpp Component Node (loadable into a component container for zero-copy intra-process transport).

Responsibility: the core ML inference engine. Loads Phi-3 mini Q4_K_M GGUF at startup via llama.cpp's C API and holds the model context warm. Handles ResolveWaypoint.srv requests. All inference runs in a rclcpp::ThreadPool worker (pool size 2) so the service server's spin thread is never blocked, preventing BT tick starvation during inference.

GBNF grammar: at startup, fetches the current room name list from /annotation_map_node via ListRooms.srv, constructs the grammar string, and initialises the llama_grammar object. On each AddRoom event (via a callback registered with /annotation_map_node), regenerates the grammar. The grammar enforces that the entire sampled token sequence forms valid JSON with a "room" field whose value is exactly one of the enumerated canonical names, not a prompt instruction but a token-sampler structural constraint.

Prompt template (simplified):

[System] You are a robot navigation assistant operating in [facility name].

Your only task is to identify which of the following locations the user wants to navigate to.

Available locations: charging_dock, kitchen, server_room, meeting_room_a, ...

Respond ONLY with valid JSON. Do not include any other text.

[User] {natural_language_command}

Temperature: 0.1. Max tokens: 256. n_ctx: 2048. n_threads: 6 (all performance cores on Orin NX).

Interfaces: service server /resolve_waypoint [ResolveWaypoint]; calls /list_rooms [ListRooms] at startup and on graph updates.

Key parameters:

- model_path: "models/phi3-mini-4k-instruct.Q4_K_M.gguf"

- n_ctx: 2048

- n_threads: 6

- temperature: 0.1

- max_tokens: 256

- timeout_ms: 3000

### 4.4 Module 3: /annotation_map_node

Type: rclcpp Node.

Responsibility: the location database. Loads room_annotations.json at startup. Serves three services. Publishes RViz2 visualisation markers. Persists runtime changes back to JSON on every AddRoom call.

Services exposed:

- GetRoomPose.srv: takes a canonical room name, returns PoseStamped. Used by the LLM resolver after a successful resolution to retrieve the metric goal.

- AddRoom.srv: appends a new room to the in-memory graph, regenerates the GBNF grammar in the resolver via callback, and serialises the updated graph to room_annotations.json. This enables a supervisor to add a new location without restarting any node; the robot learns it immediately.

- ListRooms.srv: returns all canonical names and their aliases. Called by the resolver at each service request to build the current candidate list for the LLM prompt and grammar.

Annotation graph JSON schema:

{

"rooms": [

{

"name": "charging_dock",

"aliases": ["charging station", "dock", "charger", "power point"],

"pose": {"x": 3.2, "y": -1.1, "theta": 1.57, "frame": "map"},

"tags": ["utility", "maintenance"],

"parent": "lab_floor_1"

}

],

"edges": [

{"from": "charging_dock", "to": "server_room", "relation": "adjacent"}

]

}

Edges are optional spatial relationships used for multi-hop disambiguation: when the LLM system prompt includes "charging_dock is adjacent to server_room," the model can correctly resolve "the room next to the charging dock" even without an exact alias match.

Publications: /semantic_markers [visualization_msgs/MarkerArray]: room names displayed as text markers at their annotated poses in RViz2, providing a live visual audit of the annotation graph.

### 4.5 Module 4: /bt_semantic_plugin (pluginlib plugin)

Type: BT::ActionNodeBase subclass registered as nav2_behavior_tree::SemanticNavigateToPose via pluginlib.

Responsibility: the architectural novelty. This is the only component in the existing ROS 2 / Nav2 open-source ecosystem that embeds LLM inference inside a BehaviorTree action node, loaded via Nav2's plugin mechanism, participating in BT tick timing and blackboard communication.

Ports:

- Input: command (string), the natural-language navigation command

- Output: resolved_pose (geometry_msgs/PoseStamped), the resolved metric goal

- Output: resolved_room (string), the resolved canonical room name (for logging and downstream BT nodes)

tick() logic in full:

- Get command from input port.

- Call /list_rooms synchronously (fast, ~0.1ms).

- Call /resolve_waypoint asynchronously; spin until future completes or timeout_ms elapses.

- If timeout: return NodeStatus::FAILURE. BT fallback subtree: LED pattern on /cmd_lightring + TTS "Navigation request timed out. Please try again."

- Extract calibrated_confidence from response (calibration model runs in resolver, result returned in service response).

- If calibrated_confidence ≥ threshold: write resolved_pose to blackboard output port, return NodeStatus::SUCCESS. BT continues to NavigateToPose action.

- If below threshold: publish {command, resolved_room, calibrated_confidence} to /clarification_request, return NodeStatus::FAILURE. BT fallback subtree: TTS "I'm not sure which location you mean. I know: [room list]. Could you rephrase?"

Plugin registration (plugin_description.xml):

<library path="semantic_waypoint_planner">

<class name="nav2_behavior_tree::SemanticNavigateToPose"

type="semantic_waypoint_planner::SemanticNavigateToPose"

base_class_type="BT::ActionNodeBase">

<description>Resolves NL command to Nav2 pose via on-device LLM</description>

</class>

</library>

nav2_params.yaml addition (the only change required to an existing Nav2 stack):

bt_navigator:

ros__parameters:

plugin_lib_names:

- nav2_compute_path_to_pose_action_bt_node

- nav2_follow_path_action_bt_node

- semantic_waypoint_planner   # ← one line to add

### 4.6 Complete Data Flow (End-to-End)

| Step | Actor | Operation |
| --- | --- | --- |
| 1 | Operator | Types or speaks "go to the charging dock near the fire exit" |
| 2 | /semantic_goal_node | Validates, publishes SemanticGoal.msg, calls Nav2 NavigateToPose action |
| 3 | Nav2 BT navigator | BT executor ticks SemanticNavigateToPose plugin |
| 4 | /bt_semantic_plugin | Calls /list_rooms (0.1ms), calls /resolve_waypoint |
| 5 | /llm_resolver_node | Constructs prompt + GBNF grammar; Phi-3 mini inference (~700ms); returns {"room": "charging_dock", "confidence": 0.91} |
| 6 | /llm_resolver_node | Calls /get_room_pose("charging_dock") |
| 7 | /annotation_map_node | Returns PoseStamped {x:3.2, y:-1.1, theta:1.57, frame:map} |
| 8 | /llm_resolver_node | Calibration model: raw 0.91 → calibrated 0.84; returns full service response |
| 9 | /bt_semantic_plugin | 0.84 ≥ 0.72 threshold → writes pose to blackboard, returns SUCCESS |
| 10 | Nav2 BT navigator | NavigateToPose action executes with resolved PoseStamped |
| 11 | TurtleBot4 | Navigates to charging dock via smac_planner + dwb_controller |
| 12 | ros2 bag | Logs: command, resolved room, raw confidence, calibrated confidence, latency, outcome |

### 4.7 Complete Interface Definitions

SemanticGoal.msg

std_msgs/Header header

string command

float32 confidence_threshold     # default 0.72

bool allow_approximate           # if true, nearest matching room acceptable

ResolveWaypoint.srv

# Request

string natural_language_command

string[] candidate_room_names

# Response

geometry_msgs/PoseStamped resolved_pose

string matched_room_name

float32 raw_confidence

float32 calibrated_confidence

string reasoning                 # LLM chain-of-thought, logged to ros2 bag

bool success

AddRoom.srv

# Request

string name

string[] aliases

geometry_msgs/Pose pose

string[] tags

string parent                    # optional floor/zone hierarchy

# Response

bool success

string message

ListRooms.srv

# Request

# (empty)

# Response

string[] canonical_names

string[] all_aliases             # flat list of all aliases across all rooms

### 4.8 Experimental Design

Three-condition repeated-measures study:

- Condition A (Baseline): operator uses RViz2 2D Nav Goal (metric pose input). Establishes navigation success ceiling independent of language interface.

- Condition B (Full system): grammar-constrained LLM + calibration gate at threshold 0.72.

- Condition C (Ablation, gate disabled): grammar-constrained LLM with threshold = 0.0 (all resolutions passed, no calibration gate). Isolates the gate's contribution.

Offline ablation (synthetic benchmark): Grammar-constrained vs. free-form JSON across 500 pairs. Reports: top-1 accuracy, out-of-graph output rate, false-pass rate. This directly tests the core safety claim: that grammar-constrained decoding eliminates out-of-graph outputs structurally, while free-form generation produces a measurable hallucination rate.

Participants: 5 non-expert, 2 sessions, 40 commands/session (Session 1: calibration), 20 commands/session (Session 2: evaluation). Counterbalanced condition order.

Dependent variables: task success rate (robot reaches intended room), time-to-navigation-start (command issued → robot begins moving), false-start rate (robot navigates to wrong room), operator satisfaction (Likert 1–5, four-item scale), end-to-end latency distribution (p50/p90/p99).

Evaluation metrics:

| Metric | Measurement method | Target |
| --- | --- | --- |
| Top-1 resolution accuracy | Synthetic 500-pair benchmark | ≥ 85% |
| Top-3 resolution accuracy | Synthetic 500-pair benchmark | ≥ 97% |
| Out-of-graph output rate (grammar-on) | Count / 500 benchmark runs | 0% |
| Out-of-graph output rate (grammar-off) | Count / 500 benchmark runs | Report as found |
| ECE (raw LLM confidence) | Session 2 real trials | Report as baseline |
| ECE (calibrated confidence) | Session 2 real trials | ≤ 0.08 |
| False-rejection rate | Session 2 real trials | ≤ 15% |
| False-pass rate | Session 2 real trials | ≤ 5% |
| Latency p50 / p90 / p99 | Hardware timing on Orin NX | p90 ≤ 3,000ms |

This document addresses all four rubric criteria: Domain Knowledge / Problem Statement (§1), Literature Review, 17 papers cited (§2), Design of Proposed Methodology (§3), Module Description / System Design (§4).